DISCLAIMER: This document is intended solely as template guidance for staff to use for applicants seeking a permit for an unpermitted Accessory Dwelling Unit constructed prior to 2020. It is not legal advice regarding any jurisdiction's specific policies. Local staff should consult with their city attorney or county counsel when determining the applicability of these provisions in their jurisdiction.

# Property Owner Guidance for Obtaining Permits for Unpermitted Accessory Dwelling Units and Junior Accessory Dwelling Units in the [City/County/Town of]

## Background

This handout provides guidance for property owners who want to obtain permits for Accessory Dwelling Units (ADUs) and/or Junior Accessory Dwelling Units (JADUs) that were built without permits before January 1, 2020. The information in this handout reflects amendments to California ADU and JADU law that went into effect on January 1, 2025.

In accordance with California Government Code Section 66332, the [City/County/Town] will not deny a permit for an unpermitted ADU/JADU built before 2020 solely because it does not meet building standards or state or local standards for ADUs/JADUs. However, this permitting process does not apply to buildings deemed “substandard” pursuant to Section 17920.3 of the Health and Safety Code. Thus, the [City/County/Town] may deny a permit if the ADU/JADU does not meet the standards specified in Section 17920.3 of the Health and Safety Code, which lists substandard building conditions.

[See the ADUs Amnesty Program Guide (coming soon) for guidance on general legal requirements and how to develop a legalization program.]

## Construction Date Documentation

[Per California law, local agencies may not deny a permit to ADUs and JADUs that were built without permits before 2020 and that meet the standards in Health and Safety Code Section 17920.3. The law does not explain what proof is required to show that the unit was built prior to 2020. Jurisdictions may ask applicants for documentation that the ADU/JADU existed prior to 2020. If your jurisdiction requires such documentation, provide those documentation requirements here.]

## Health and Safety Standards

The [City/County/Town of] [Building/Community Development Department] may deny a permit for an unpermitted ADU or JADU constructed before January 1, 2020, if the ADU or JADU does not comply with all of the health and safety standards listed below. However, the [City/County/Town] will approve any necessary permits to correct any violations of health and safety standards. Once the building is no longer substandard, the unpermitted ADU or JADU will qualify for a permit.

In order for an ADU or JADU to obtain permits they must meet certain habitability and safety standards as defined in Section 17920.3 of the Health and Safety Code and provided in Table 1, below.

These standards generally include, but are not limited to:

- Sanitation facilities including a working bathtub/shower, kitchen sink, hot and cold running water, toilet and connection to a sewage system.

- Electrical, heating/ventilation and plumbing systems that work and have been maintained in a good and safe condition.

- Livable conditions without mold or infestations of insects or rodents.

- Adequate exit facilities.

These standards also require that certain conditions, if present, would need to be corrected; these conditions generally include, but are not limited to:

- Deteriorated walls, windows, doors, ceilings, roofs, floors, foundations, chimneys or other structural elements.

- Electrical, mechanical or plumbing systems that do not conform with applicable laws and regulations in effect at the time of their installation.

- Improper construction, materials or structural hazards.

- Fire or other hazards.

Applicants cannot be required to pay impact fees or connection or capacity charges except when utility infrastructure is required to comply with health and safety standards.

## Inspecting the ADU/JADU

Property owners applying for a permit for an unpermitted ADU or JADU are advised that upon receipt of an application for a permit for the unit, the [City/County/Town] [Building/Community Development Department] may inspect the unit and provide recommendations for achieving compliance with health and safety standards. Prior to submitting a permit application, the property owner may obtain a confidential third-party code inspection from a licensed contractor, to determine the unit’s existing condition or potential scope of building improvements before submitting an application for a permit. Table 1 lists the detailed requirements that the [City/County/Town] [Building/Community Development Department] or third-party licensed contractor will inspect.

After the [City/County/Town] Building Department has issued a permit for an unpermitted ADU or JADU constructed before January 1, 2020, the unit may be subject to inspection by a [City/County/Town] official to confirm that the unit complies with the more stringent building standards contained in the State Building Standards Code, pursuant to Article 1 (commencing with Section 17960) of Chapter 5 of Part 1.5 of Division 13 of the Health and Safety Code. The owner can request a delay in enforcement of these standards.

### Table 1: Checklist for Local Agency or Third-Party Inspection

This table describes the conditions that must be corrected prior to being issued a permit.

| Health and Safety Standards (California Health and Safety Code Section 17920.3) | Needs Correction? (Yes/No/Comments) |
| --- | --- |
| Sanitation (Section 17920.3(a)) Inadequate sanitation shall include, but not be limited to: Lack of, or improper water closet, lavatory, bathtub or shower. Lack of, or improper kitchen sink. Lack of hot and cold running water to plumbing fixtures. Lack of minimum amounts of natural light and ventilation. Dampness of habitable rooms. Infestation of insects, vermin, or rodents. Visible mold growth. General dilapidation or improper maintenance. Lack of connection to required sewage disposal system. Lack of adequate garbage and rubbish storage and removal facilities. |  |
| Structural (Section 17920.3(b) & (o)) - Structural hazards shall include, but not be limited to: Deteriorated or inadequate foundations. Defective or deteriorated flooring or floor supports. Flooring or floor supports of insufficient size to carry imposed loads with safety. Members of walls, partitions, or other vertical supports that split, lean, list, or buckle due to defective material or deterioration. Members of walls, partitions, or other vertical supports that are of insufficient size to carry imposed loads with safety. Members of ceilings, roofs, ceiling and roof supports or other horizontal members which sag, split, or buckle due to defective material or deterioration. Members of ceilings, roofs, ceiling and roof supports, or other horizontal members that are of insufficient size to carry imposed loads with safety. Fireplaces or chimneys which list, bulge, or settle, due to defective material or deterioration. Fireplaces or chimneys which are of insufficient size or strength to carry imposed loads with safety. Inadequate structural resistance to horizontal forces. |  |
| Nuisance (Section 17920.3(c)) The unit has a nuisance. |  |
| Electrical (Section 17920.3(a)(10) & (d)) Does the unit have: Wiring that did not conform to all applicable laws in effect at the time of installation. Wiring that is not in good and safe condition and working properly. Lack of required electrical lighting. |  |
| Plumbing (Section 17920.3(e)) Plumbing which did not conform with all applicable laws and regulations in effect at the time of its installation. Plumbing which has not been maintained in good or safe condition, or has cross-connections and siphonage between fixtures. |  |
| Mechanical (Section 17920.3(a)(6), (a)(7) & (f)) Mechanical equipment, including vents, which did not conform with all applicable laws and regulations in effect at the time of its installation, has not been maintained in good and safe condition, or is currently not in good and safe condition and is not working properly. Lack of adequate heating. Lack of, or improper operation of required ventilating equipment. |  |
| Weather Protection (Section 17920.3(g)) Faulty weather protection, which includes, but is not limited to, the following: - Deteriorated, crumbling or loose plaster. - Deteriorated or ineffective waterproofing of exterior walls, roof, foundations, or floors, including broken windows or doors. - Defective or lack of weather protection for exterior wall coverings, including lack of paint, or weathering due to lack of paint or other approved protective covering. - Broken, rotted, split, or buckled exterior wall coverings or roof coverings. |  |
| Fire Hazards (Section 17920.3(h)) The building, structure, or portion thereof, has a device, apparatus, equipment, combustible waste, or vegetation which is in such a condition as to cause a fire or explosion or provide a ready fuel to augment the spread and intensity of fire or explosion arising from any cause. |  |
| Construction Materials (Section 17920.3(i)) Materials of construction used in the unit are not specifically allowed by the Health & Safety Code. The unit’s materials of construction have not been adequately maintained in good and safe condition. |  |
| Fire, Health or Safety Hazards (Section 17920.3(j)) An accumulation of weeds, vegetation, rubbish, dead organic matter, debris, garbage, offal, rat harborages, stagnant water, combustible materials, or similar materials or conditions constitute fire, health, or safety hazards on the premises. |  |
| Maintenance (Section 17920.3(k)) The building or portion thereof is determined to be an unsafe building due to inadequate maintenance, in accordance with the latest edition of the Uniform Building Code. |  |
| Exit Facilities (Sections 13143.2 & 17920.3(l)) Lacking adequate exit facilities, excepting those buildings or portions thereof whose exit facilities conformed with all applicable laws and regulations at the time of the unit’s construction. Exit facilities that have not been maintained and increased in relation to any increase in occupant load, alteration or addition, or any change in occupancy. Lacking structural fire safety and fire-resistant exits, provided that the ADU/JADU is located in a multiple-story structure that existed on January 1, 1975, in which rental units are situated above the ground floor. |  |
| Fire Protection (Section 17920.3(m)) Lacking fire-resistive construction or fire-extinguishing systems or equipment, excepting a building, structure, or portion thereof which conformed with all applicable laws and regulations at the time of its construction. Lacking fire-resistive integrity and fire-extinguishing systems or equipment that have been adequately maintained and improved in relation to any increase in occupant load, alteration or addition, or any change in occupancy. |  |
| Improper Use (Section 17920.3(n)) Buildings, structures, or portions thereof occupied for living sleeping, cooking, or dining purposes not designed or intended to be used for those occupancies. |  |
| Room and Space Dimensions (Section 17920.3(a)(9)) Room and space dimensions are less than required by code. |  |

## For More Information

Property owners interested in obtaining a permit for an unpermitted ADU or JADU may contact the [City/County/Town of] [Building/Community Development Department] at [phone number/email address] with any questions about the process.

The list of substandard conditions in Section 17920.3 has been reorganized in Table 1 for ease of use.